# Thrust Stand — Complete Build Guide

Project Icarus | Options A / B / C | v6 — April 2026

Project Icarus — Ion Wind Aircraft

Fig 1 — ZVS Flyback Module

Fig 2 — Arduino Nano Every

Fig 3 — Emitter wire with corona glow

Fig 4 — NACA airfoil collector foil

# Configuration Options

| Option | Collector | Gap | Thrust | Cost |
| --- | --- | --- | --- | --- |
| A (Starter) | NACA airfoil | 20mm | 169 mN | ~EUR64 |
| B (Tandem) | NACA + tandem ducted | 8mm | 1627 mN | ~EUR80 |
| C (+Steel Wool) | Wire-to-mesh + steel wool | 20mm | 234 mN | ~EUR69 |

WARNING: This system generates 20-50kV. Never touch the HV wires when powered. CORONA = soft violet glow (good). ARCING = bright white sparks (bad). Increase gap or reduce voltage if arcing occurs.

# PART 1: Bill of Materials (BOM)

| Component | Supplier | Price |
| --- | --- | --- |
| ZVS Flyback Module | reichelt.de / Amazon | EUR10 |
| Arduino Nano Every | reichelt.de | EUR10 |
| Stainless wire 0.1mm x10m | Amazon.de | EUR6 |
| Copper foil 0.05mm x5m | Amazon.de | EUR12 |
| Acrylic sheet 3mm 20x30cm | Amazon.de / OBI | EUR10 |
| M3 threaded rod + nuts | Amazon.de | EUR6 |
| HX711 + Load Cell 5kg | Amazon.de | EUR12 |
| Resistors 10M + 1M | Amazon.de / reichelt | EUR1 |
| 18650 holder x2 + batteries | Amazon.de | EUR8 |
| Aluminum mesh + steel wool | Supermarket | EUR5 |

# PART 2: Know Your Components

## 2.1 ZVS Flyback Module

Fig — 2.1 ZVS Flyback Module

4 terminals: 2 input (VCC+/GND-) and 2 HV output (HV+/HV-). Takes 5-12V DC, outputs 20-50kV AC.

## 2.2 Arduino Nano Every

Fig — 2.2 Arduino Nano Every

A0=HV divider reading, GND=common ground, 5V=power. Connect to PC via USB-mini.

## 2.3 Emitter Wire

Fig — 2.3 Emitter Wire

Very thin wire (0.1-0.2mm). Thinner = lower onset voltage. Stainless steel 0.1mm = most practical.

## 2.4 Collector Foil

Fig — 2.4 Collector Foil

Copper or aluminum, smooth, no sharp edges. Sharp edges = arcing instead of corona.

# PART 3: Step-by-Step Assembly

## STEP 1: Identify ZVS terminals

Fig — STEP 1: Identify ZVS terminals

INPUT: 2 small terminals — GND (ground) and VCC (5-12V positive)

HV OUTPUT: 2 large terminals — HV+ and HV-

The HV output wires go directly to the electrodes — DO NOT touch when powered!

## STEP 2: Connect ZVS to power supply

Fig — STEP 2: Connect ZVS to power supply

1. Connect ZVS VCC to positive of power supply (red wire of 18650 holder)

2. Connect ZVS GND to negative/ground of power supply (black wire)

3. Turn on power — a high-pitched hum = ZVS is working

4. If module gets hot >50C, turn off immediately — check polarity

## STEP 3: Mount acrylic structure

Fig — STEP 3: Mount acrylic structure

1. Cut acrylic: 2x 15cmx3cm (vertical posts) + 2x 20cmx3cm (top and base)

2. Drill 4x 3mm holes in each post (for M3 screws)

3. Assemble sandwich: base + left post + spacers + right post + base

4. Do NOT overtighten — you need to adjust the gap later

## STEP 4: Stretch emitter wire

Fig — STEP 4: Stretch emitter wire

1. Thread 0.1mm wire between the two posts, centered

2. Stretch until taut like a guitar string — but not to the point of snapping

3. Wire must not touch anything, completely stretched and straight

4. Secure with electrical tape at the ends — do NOT use anything conductive

## STEP 5: Mount collector foil (Option A)

Fig — STEP 5: Mount collector foil (Option A)

1. Cut copper/aluminum strip ~2cm wide x 15cm long

2. Shape around a tube/cylinder to form an airfoil curve

3. Place collector parallel to the emitter wire

4. Gap: 20mm initial (adjust later based on results)

## STEP 5C: Wire-to-mesh + Steel Wool (Option C)

Fig — STEP 5C: Wire-to-mesh + Steel Wool (Option C)

1. Cut aluminum mesh ~5cm x 50cm (mosquito net type)

2. Stretch mesh over acrylic — stays flat but with texture (visible holes)

3. Cut steel wool #0000 or #00 (~5cm x 50cm)

4. Place steel wool ON TOP of the mesh — it stays suspended and makes electrical contact with the mesh

5. Connect mesh+steel wool to HV- wire (ground/cathode)

6. 3D texture = ~1.8x more thrust

7. Gap: 20mm (same safe as Option A)

## STEP 6: Verify gap

Fig — STEP 6: Verify gap

1. Use a ruler or caliper to measure the gap

2. Gap < 15mm = ARCING (sparks) instead of corona

3. Gap > 40mm = very weak thrust

4. 20mm = optimal starting point at 35kV

CORRECT: soft violet glow. WRONG: bright sparks = increase gap!

## STEP 7: Connect HV wires

Fig — STEP 7: Connect HV wires

1. HV+ (ZVS) → emitter wire (anode/positive)

2. HV- (ZVS) → collector foil (cathode/negative)

3. Use flexible copper wire, good solder joint — bad connection = arcing

## STEP 8: Mount HV voltage divider

Fig — STEP 8: Mount HV voltage divider

1. Resistor 10M: connect to one HV output wire

2. Resistor 1M: connect in series (other side of 10M to the other HV wire)

3. Junction point (where the two resistors meet) → Arduino pin A0

4. Voltage at junction = Total HV Voltage / 11 (ex: 2.7V reading = ~30kV real)

## STEP 9: Mount pendulum

Fig — STEP 9: Mount pendulum

1. Tie nylon string/cord (20-30cm) to the top of the electrode structure

2. Hang from a pivot (pencil across two tables, or a nail in a wall)

3. Pendulum must swing freely in a horizontal plane

4. Electrode deflects toward the collector when HV is on

## STEP 10: Calibrate pendulum with known weights

Fig — STEP 10: Calibrate pendulum with known weights

1. With HV OFF, hang known weights (M3 nuts ~1.5g each) on the pendulum string

2. Measure deflection for each weight:

- 0 = 0mm

- 1x M3 nut (~1.5g) = ~3mm

- 5x M3 nuts (~7.5g) = ~15mm

3. Save this table — you will use it to calculate real thrust

## STEP 11: Connect Arduino

Fig — STEP 11: Connect Arduino

1. Divider junction point → Arduino pin A0

2. Arduino GND → ZVS GND (common ground)

3. Arduino → PC USB. Open Arduino IDE, Serial Monitor at 9600 baud.

Code:

void setup(){Serial.begin(9600);}

void loop(){

int raw=analogRead(A0);

float kV=raw*11.0/1023.0;

Serial.print("HV: ");Serial.print(kV,1);Serial.println(" kV");

delay(500);

}

## STEP 12: First HV test

Fig — STEP 12: First HV test

1. Turn on ZVS, increase voltage slowly

2. Watch the pendulum — it starts deflecting toward the collector

3. Record: HV voltage (kV), maximum deflection (mm), anything unusual

4. CORONA = soft violet glow (good)

5. ARCING = bright white/blue sparks + loud hum = increase gap or reduce voltage

## STEP 13: Calculate real thrust (mN)

Fig — STEP 13: Calculate real thrust (mN)

Formula: Thrust(mN) = (deflection_mm/1000) x (weight_g x 9.81) x (L_gap/L_pendulum)

Example: L_pendulum=250mm, deflection=5mm, weight=50g:

Thrust = (5/1000)x(50x9.81)x(25/250) = 1.2 mN

# PART 4: Suppliers

| Supplier | Products | Notes |
| --- | --- | --- |
| reichelt.de | Electronics, Arduino, HX711 | Best for electronics. EUR5.90 DE shipping. |
| Amazon.de | Wire, foil, hardware | Fast delivery. Consumables. |
| Pollin.de | Cheap components, refurb | EUR4.95 shipping. |
| Conrad.de | Lab equipment, power supplies | EUR5.99 shipping. |
| eBay.de | Wire, foil, used flybacks | DE sellers. Low prices. |

# PART 5: Troubleshooting

Low thrust: check wire is taut, increase voltage, collector is smooth, reduce gap (min 10mm at 35kV).

Arcing: increase gap, reduce voltage, check collector has no sharp edges.

ZVS not working: check polarity, supply voltage 5-12V/2A min, check for short circuits.